Project Title

To develop a 50 gm scale process of nitrobenzene catalytic hydrogenation using a commercial catalyst to make aniline.

Objectives

- To develop a 50-gram batch scale process for aniline synthesis using nitrobenzene catalytic hydrogenation using the commercial catalyst.

- To optimize all the process parameters such as temperature, pressure, hydrogen feed rate, catalyst loading, coolant temperature, and its flow rate. Efforts will be made to achieve more than 99 % conversion.

- To do the catalyst recyclability study.

- To develop a suitable separation protocol for aniline from the reaction mass.

- Reaction calorimeter study to determine heat rate due to exothermic reaction and heat rate sensitivity with the process parameters with solvent and without solvent.

Executive summary

In this proposal, a lab-scale two-litre batch reactor scale nitrobenzene catalytic hydrogenation process to produce aniline is being proposed. POC study at 10 gm nitrobenzene scale has already been done using ethanol as solvent and as catalyst. For the POC study, we are able to get more than 99 % reaction conversion while varying the process parameters. Nitrobenzene catalytic hydrogenation gives aniline as the main product and water as by-product (Figure 1); there are few intermediates reported, which disappear when the reaction completes. In this proposal, we will develop and optimize the nitrobenzene hydrogenation process at a two-litre reactor scale. As this is a highly exothermic reaction, we will optimize the hydrogen feed rate, coolant temperature, and coolant flow rate in such a way that the exothermic heat can be handled without any significant temperature rise of the reactor and without any runaway situation.

Figure 1: Nitrobenzene catalytic hydrogenation reaction scheme

We will use the commercial catalyst for this process, and to reduce the cost of the catalyst we will do the catalyst recyclability study also. We will also make efforts to make this process as solvent-free process. We will develop the separation protocol for the separation of the catalyst and aniline from the reaction mass after the reaction completion. Aniline is an industrially important molecule, and there is huge import of it. Heat rate data due to nitrobenzene hydrogenation exothermic reaction and heat rate sensitivity with the process parameters with solvent and without solvent is not available in the literature; we will generate this information using the Reaction calorimeter. There is no indigenous technology provider for aniline production. Hence, there is a good opportunity to develop this process and technology and give them to Indian chemical industries for the large-volume production, so that aniline import can be reduced or made to zero.

Total Project Cost (Rs. in lakhs): 150

- Capital     90

- Recurring   60

Project Duration (In months):  24

Technology Readiness Level

- Current

Most of the industrial aniline production is done by nitrobenzene catalytic hydrogenation, which is the simplest route. Currently, In India, there is no technology provider for nitrobenzene hydrogenation for aniline production. Gujarat Narmada Valley Fertilizers & Chemicals Ltd and R K Synthesis are the only aniline producers in India based on the technology provided by Chematur Engineering AB, Sweden.

- Targeted

A lab-scale nitrobenzene catalytic hydrogenation process using a commercial catalyst for two litre reactor scale will be developed and optimized. Efforts will be made to achieve minimum or nil solvent quantity for this process. Nitrobenzene hydrogenation is a highly exothermic reaction; hence, for the safe handling of the process; hydrogen addition rate (ml/min), coolant temperature and coolant flow rate will be optimized and will be chosen in such a way that heat generated can be handled easily with the available cooling and without any run-away conditions. Reaction calorimeter will be used for the heat rate data generation due to exothermic reaction. To reuse the catalyst its recyclability study will also be done. Understanding and learning from the two-litre developed process will help to Scale-up the process at pilot and commercial scales.

Preliminary Results

For the POC study, nitrobenzene catalytic hydrogenation study at 10 gm nitrobenzene scale has been done with solvent and catalyst. Experiments with varying reactor temperature, reactor pressure and substrate amount were also conducted.

Technology Readiness Level (TRL)

At the initiation of the project: 3

At the completion of the project: 4 or 5

Deliverables

- A safe, cost-effective, solvent less or with minimum solvent nitrobenzene catalytic hydrogenation process to produce aniline at 50 gm batch scale using the commercial catalyst.

- Optimization of all the process parameters such as temperature, pressure, hydrogen feed rate, catalyst loading, reaction time, etc.

- Catalyst recyclability study.

- Separation protocol for aniline from the reaction mass.

- Reaction calorimetry study of nitrobenzene hydrogenation reaction and its heat rate data.

- Currently, we have this process at the POC stage at TRL 3; efforts will be made to achieve TRL 4 or 5 at the end of this project.